COBIFIVE Datasheet

Five core quantum-inspired Ising solver chip

Version: 2024.11.19

Contents

1. Overview	1

2. Pin Configuration	2

3. Pin Function	3

4. Supply Voltage and Current	4

5. Chip Block Diagram	4

6. Input format	6

7. Output Format	8

8 Reset Timing and Control	9

9. Application Information	9

9.1 Write Timing and Control	9

9.2 Read Timing and Control	10

Multiple boards, multiple chips, and multiple core operation example	11

Appendix A	13

Input data (52 x 51 x 4b)	13

Expected output from chip (69 bits)	13

# 1. Overview

COBIFIVE Ising Solver Chip

AXI Interface

- >50MHz AXI clock frequency

- Asynchronous read-write

- 16-bit write data bus

- 1-bit read data

- *_LAST signals included in the AXI interface

Input/Output Pads

- 25 input/output digital pins

  - Input pin configuration: Schmitt trigger

  - Output pin configuration: Hysteresis and pull-down, 12mA drive current per pin

Features

- Manufactured in 28nm

- 0.9V core voltage, 1.8V IO voltage

- Input data:	52 x 51 hex numbers = 52 x 51 x 4 = 10,608 bits

- Output data: 	69 bits

  - 15b (best_ham) + 46b (best_spin) + 4b (core id) + 4b (problem_id)

  - First output: best_ham

  - Last output: problem_id

- Programmable on-chip clock frequency ranging from 150MHz to 2GHz

# 2. Pin Configuration

Fig. 2.1 illustrates the bonding diagram of COBIFIVE which is using QP-QFN48-6MM-.4MM package type as shown in Fig. 2.2.

Fig. 2.1. COBIFIVE chip bonding diagram

Fig. 2.2. COBIFIVE chip package diagram (QFN 48, 6mm x 6mm, 0.4mm pitch)

# 3. Pin Function

Table 3.1 describes the Pin Functions of COBIFIVE

| Pin |  | I/O | Description | Activation Type |
| --- | --- | --- | --- | --- |
| Name | Number |  |  |  |
| Power/ Ground |  |  |  |  |
| VDD1P8 | 6,19,31,42 | PWR | I/O power supply 1.8V | NA |
| VSS1P8 | 1,13,30,36 | PWR | I/O power supply 0V | NA |
| VDD0P9 | 4,9,16,21,28,33,40,45 | PWR | Core power supply 0.9V | NA |
| VSS0P9 | 5,8,18,29,32,41,44 | PWR | Core power supply 0V | NA |
| CLK, Reset |  |  |  |  |
| CLK | 27 | Input | AXI clock used for both asynchronous read and write | Active High |
| RESETB | 24 | Input | Active low reset for entire chip | Active Low |
| AXI Data Transmission Interface |  |  |  |  |
| S_DATA<0:15> | 2,3,7,10,11,12,14,15,23,37,38,39,43,46,47,48 | Input | Write Data, S_DATA<15> is MSB | Active High |
| S_VALID | 34 | Input | Indicates S_DATA input is valid | Active High |
| S_LAST | 25 | Input | Toggled active for 1 CLK cycle on the last S_DATA stream in | Active High |
| S_READY | 35 | Output | When active, COBIFIVE is ready to receive write data | Active High |
| M_DATA | 26 | Output | Read Data | Active High |
| M_VALID | 17 | Output | COBIFIVE indicating M_DATA is valid | Active High |
| M_LAST | 22 | Output | Toggled active for 1 CLK cycle on last M_DATA stream out | Active High |
| M_READY | 20 | Input | When active, chip will load read data | Active High |

Table 3.1 Pin functions

# 4. Supply Voltage and Current

Table 4.1 shows absolute maximum ratings over operating free-air temperature range (unless otherwise noted).

| Rating | Value | Unit |
| --- | --- | --- |
| VDD1P8 Voltage | 1.8 | V |
| VSS1P8 Voltage | 0 | V |
| VDD1P8 Current | 20 (max) | mA |
| VSS1P8 Current | 20 (max) | mA |
| VDD0P9 Voltage | 0.9 | V |
| VSS0P9 Voltage | 0 | V |
| VDD0P9 Current | 140 (max) | mA |
| VSS0P9 Current | 140 (max) | mA |
| Digital I/O Voltage | 1.8 | V |
| Digital I/O Current per pin | 12 (max) | mA |

Table 4.1 Supply voltage and current consumption

# 5. Chip Block Diagram

Fig. 5.1 depicts the high level block diagram of the chip.

Fig. 5.1 COBIFIVE Block Diagram

# 6. Input format

There are four types of functional bits shown in Fig. 6.1, including calibration bits (red), SHIL (blue), control bits (yellow), dummy bits (green), short bits (gray) and data (white). The chip is configured and operates under various conditions by assigning different values to the above functional bits. In order to comply with the 16-bit AXI transaction requirements, two additional random data elements are inserted at the beginning of each line. For the actual input data, it is recommended to replace these random values with zeros.

Fig. 6.1 Input Graph Format

| Control bits | Width | Description | Range Values | Suggested default |
| --- | --- | --- | --- | --- |
| sample_time | 16b | Parameter in controller, time duration between two sample signal, would better if greater than 8C which is used for pre-silicon verification | 0x0000 ~ 0xFFFF | 0x00FD |
| weight_time | 16b | Parameter in controller, time duration between two weight_enb (weight enable) signals, usually the same as shil_time | 0x0000 ~ 0xFFFF | 0x0003 |
| shil_time | 16b | Parameter in controller, time duration between two shil_enb (shil enable) signal, usually the same as weight_time_off | 0x0000 ~ 0xFFFF | 0x000F |
| rosc_time | 16b | Not used | 0x0000 ~ 0xFFFF | 0x0003 |
| max_fails | 16b | Parameter in accelerator, a static sampling parameter, should be no less than 1 | 0x0000 ~ 0xFFFF | 0x001F |
| sample_delay | 16b | Parameter in accelerator, delay parameter from Ising core to accelerator | 0x0000 ~ 0xFFFF | 0x00FF |
| dco_freq | 4b | Frequency setting for dco, used for configuring different dco frequency - Chip will only function correctly when dco_freq = 0x0000 or it will result in intermittent stalls | 0x0000 ~ 0x000F | 0x0000 (USE THIS VALUE ONLY) |
| problem_id | 16b | The id of problems, special meaning for the highest bit - problem_id[15] = 1’b1 -> bypass gradient search (use 0 as default) | 0x0000 ~ 0xFFFF | 0x0000 ~ 0x00FF |

Table 6.1 Last row control bits explanation

Write Text File Conversion (Hexadecimal to Binary)

Here is an example of how the 4-Character Hexadecimal data is written to the chip through the AXI stream bus protocol:

4-Character Hexadecimal data = 000A

S_DATA<15:0> = 0000 0000 0000 1010

0       0       0        A

OR

S_DATA <15> = 0

S_DATA <14> = 0

S_DATA <13> = 0

S_DATA <12> = 0

S_DATA <11> = 0

S_DATA <10> = 0

S_DATA <9> = 0

S_DATA <8> = 0

S_DATA <7> = 0

S_DATA <6> = 0

S_DATA <5> = 0

S_DATA <4> = 0

S_DATA <3> = 1

S_DATA <2> = 0

S_DATA <1> = 1

S_DATA <0> = 0

The following example graph demonstrates how hexadecimal bits are grouped to form 16-bit segments and transmitted via the AXI interface. Consider the last three rows from Fig. 6.1 Input Graph as an example. These rows correspond to data, calibration, short, and control bits, respectively. In reality, the input data will transfer from the first row to the 51st row.

Fig. 6.2 Last two rows’ data from Fig. 6.1 Input Graph Format

Figure 6.3 demonstrates how the input graph data is transferred to COBIFIVE through the AXI interface. The transfer always begins from right to left for each row, as illustrated in Figure 6.4.

Fig. 6.3 Hex Bits Transition Via 16-bit AXI

Fig. 6.4 Sequential Data Transfer Across Rows with Bit Mapping

# 7. Output Format

COBIFIVE outputs 69 bits when each core completes the computation. The breakdown of the output bits from left to right is:

Best_Ham[14:0], Best_Spins[45:0], Core_ID[3:0],  Problem_ID[3:0] (only last hex digit)

Here’s an example output bit stream.

000000000000000RRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRR10001000

R: random 1’s and 0’s, changes every time

# 8 Reset Timing and Control

Table 8.1 provides the definition, typical values and maximum clock frequency.

| Pin | Description | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| CLK | AXI Write and Read Clock Frequency (tcyc = 1/fBus) | fBus | dc | 125 | 200 | MHz |

Table 8.1 AXI clock frequency

RESETB will be activated low to initialize COBIFIVE at the beginning, and then set to high while waiting for the next rising edge of CLK.

Fig. 8.1 COBIFIVE Startup/Reset

# 9. Application Information

## 9.1 Write Timing and Control

A write pattern within the COBIFIVE consists of 663 writes of 16 bits (S_DATA is 16 bits). At the last write cycle, S_LAST will be toggled high. A text file similar to Fig. 6.1, consisting of 663 lines of data with each line containing 16 bits in hexadecimal, represents the written pattern.

Data and signals to the COBIFIVE are to be switched on negative CLK edges to prevent setup and hold time violations.

Writing to the chip occurs asynchronously of reading. Writing can occur during reading.

Fig. 9.1.1 Writing Directly After Reset

Fig. 9.1.2 Write After Waiting for S_READY High

Fig. 9.1.3 Write Directly If both S_READY and S_VALID Stays High

Fig. 9.1.4 No Write If S_READY Goes Low

NOTE: After S_LAST is toggled, the COBIFIVE may or may not be able to accept more data. If both S_VALID and S_READY stays HIGH, continue to the next write pattern. Otherwise, if S_READY goes LOW after S_LAST is toggled, wait until S_READY goes HIGH again. Both scenarios are shown in “Writing Directly After Reset” and “Write After Waiting for S_READY High”. “Write Directly If both S_READY and S_VALID Stays High” shows the continuous write mode. “No Write If S_READY Goes Low” shows the stop writing mode.

## 9.2 Read Timing and Control

A read pattern within the COBIFIVE consists of 69 writes of 1 bit (M_DATA is 1 bits). At the 69th write, M_LAST will be toggled high. Read 0 is the first line of the text file. The last line is Read 68.

Data and signals from the COBIFIVE are to be recorded on negative CLK edges to ensure data stability.

Reading from the chip occurs asynchronously of writing. Reading can occur during writing.

Fig. 9.2.1 Read After Reset

Fig. 9.2.2 Read After Waiting for M_VALID

Fig. 9.2.3 Read If M_VALID Stays HIGH

Fig. 9.2.4 Read If M_VALID Goes Low

NOTE: After M_LAST is toggled, the COBIFIVE may or may not be able to read more data. If both M_VALID stays HIGH, continue to the next read-out pattern. Otherwise, if M_VALID goes LOW after M_LAST is toggled, wait until M_VALID goes HIGH again. Both scenarios are shown in “Read After Reset” and “Read After Waiting for M_VALID”. “Read If M_VALID Stays HIGH” shows the continuous write mode. “Read If M_VALID Goes Low” shows the stop writing mode.

## Multiple boards, multiple chips, and multiple core operation example

Fig. 9.4 Multiple Chips controlled by FPGA

# Appendix A

## Input data (52 columns x 51 rows x 4b)

All 0 weight graph example		                                      All +7 weight graph example

0000000000000000000000000FF0000000000000000000000000

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF00

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0F0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FF0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000F0FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0000000000000000000000000FF0000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

0000000000000000000000000FF0000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

Random weight graph example

0000000000000000000000000000000000000000000000000000

000000000980000090070000000B000000000209003070002000

00000000E060D07022800000000607080700000CB00000635000

00003000500C004070CE0000000B2D005020002A000000000000

0000000040E020C02E000006C0000C0080000000E00750A00CA0

000200C0000000060AA500300000D0D0000007B0000070000800

00000000000000A00030070A0009B60000B0030D000000000000

0000000000000000000000050000000000000007003000009000

00000D00000004700A007000000000088006002D0200B5800000

0000000000D07BC0000E0B0000000000000F0070D000000A0000

0000F080000909D60042700B00000407C0000C0000000000B020

000900002000000D5C3B0200800200600B00000000A0000F0400

00000000CE0902000000000040000C0060009E3000000E460090

000903000600C0900074007000000DD000000E00000000C86000

000000000D0B00570A0424F000000040A7003000000000000400

000CA00000D00000000FF00B000000F0000000000B0000000500

0000C000000300E0C00E092000000E0000700000020680000000

0000282D0000C04000000000000C000A0000000B0000E0E0B000

0000B00009B0F00D00000F00000000800005420020D008000000

000B00D006000400004040000000000000C0500700008500C000

000005D070030000000073050000008000400000000000000000

000200009060000A0000000000030000066000C0C0B20F000000

000000C200E0900000060500000A00700F80E0E06E0000000000

000000000F000C0000500000000006B00000B060000000900000

00000000090039080000D00A000000E002000000E00000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00000502040000B0000004CB00000000F006EA00000700000A00

000B000080050000000000A00000F000000000B0000809000000

0000E000BF000E0E000300000000E0D0E9C000C000000DD00000

000000082002FA0070000000000000D0040C60AE00950008AC00

00000056080A0090C00000623000030500908000030090D00000

00060EE00000007000000000000500002000000B008D00000700

0000000000002B00000004000000000B0E0000D4090000000400

000000E000000A008000000000050EC09400000040E0000D3550

00003E04000000000D090CB00000009C05000000D00900408000

000C000000000000E000000A000000000C00000B0390F0600500

000000000008000000F70000D0000E0000200B00050003000000

00005B00000200000000900000000000000A0000000000069500

00000A0000B00005D00B00400000030390F0009D000060000000

000080000300000A00099000000040000A090000000600000000

0009C4006000D000000800020000900200F62000220000B03070

00050000D00000900903900F200000007C000040000C0E000EB0

00000004000E0609000B000FC00E0700000000800A09000000A0

000000000400700070005000500A6D0095000000000A070C0000

000D0B0000300006060040D7000877F090000000000000000800

000000000B00000B0002000000000030050D0000000000300000

0006000000F00C000C0E0400000073005AE90300000000700000

0000300500000B80004000080000C0F00B02B0000000060D0E00

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

## Expected output from chip (69 bits)

Best_Ham[14:0], Best_Spins[45:0], Core_ID[3:0],  Problem_ID[3:0]

All 0 weight graph example

Take core 0 as an example when problem_id[15] = 1’b0

000000000000000    1111111111111111111111111111111111111111111111    0000    0001

Best_Spins[45:0] should look like random 0’s and 1’s

All + 7 weight graph example

Take core 0 as an example when problem_id[15] = 1’b0

100011101100110    0000000000000000000000000000000000000000000000    0000    0001

Best_Spins[45:0] should be all 0’s

Random weight graph example

(energy might be differ but similar to -650 as max_fails is set to tb 2 for boosting the simulation speed)

Take core 0 as an example when problem_id[15] = 1’b0

111110101110110    xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx  0000    0001

(around -650)	        Best_Spins[45:0] will be random since it is an analog circuit

# Appendix B

## Input data examples (52 columns x 51 rows x 4b) including random, all +3, all +2, and all +1 graphs, for additional testing and verification.

All +3 weight graph example

0000000000000000000000000000000000000000000000000000

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0007777777777777777777777FF7777777777777777777777770

0000000000000000000000000FF0000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

All +2 weight graph example                                                All +1 weight graph example

0000000000000000000000000000000000000000000000000000

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0005555555555555555555555FF5555555555555555555555550

0000000000000000000000000FF0000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

000FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF0

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0003333333333333333333333FF3333333333333333333333330

0000000000000000000000000FF0000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

Random weight graph 1 example                                               Random weight graph 2 example

0000000000000000000000000000000000000000000000000000

0000070B00BA8000A504E2090000060C000BC60003000C300700

00000CE06040B60004300006000B000B00800280F00000000000

000000F00006F004C0000000800B0000030200000A0940000DF0

00005C000009A0060DCE0002600002E03D0000A0080302600000

0000000000A00000000C000000000E5D00040089007070000420

000003000D0000DE0F00E00020000E0B050000000E0000000B00

00000300A0E9F02A529300F0000BB4E0003C0B0000AA00057620

0007005E200C03700000D00E400E530000002069B0C0F0000080

00009CFF00007DE00D0B07C2000A000A40B00000050000000020

000A000090000C00000002B05000000B02000075000007000000

00004000000000000600000B00000D00400202A0040040000720

000F705008D0000070E0300F00060000000009000B00F020A020

00000BF002000770000000F00000F06008800000400800009000

000B0D0704900AB000040B000000000C005A5000900D000B0000

000B0000785040000005000040030D00BAF506000000000000F0

0000A08C99000004C20400C0000F00000000003A0F0B03B00800

00080D00000F0B00000070E000000300000024B00E0D0A000000

0000700009E6F000B0050060F000660000000009005000000340

00080000006B3E00400090200000003008C00003009C94B00400

000929F807640F003000F0000000003000000000D0C0200070D0

00000050E60A6FD7CF028040000450000A0D090000004E0E0000

0000C000B00000000080000D0000000800E00300000000E00000

000D000066A603A006500000000000B0B2004000050B000D3000

000000000CB400D00600000D0000000BE6B8B050000E00000BD0

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000B0008F0AF0D60008300000007000900000A4089F09400B0D0

000B000009007455B0000E00E0009000EF805BD05E03005BE070

00000F0E400000800B40C800200E0080000D00080000A0A08000

000066DC0068C00003C20000000047000D0000BF088B90000040

0000009000C00700A00000000000407B00200500000060000900

00000009720004006020000E000E0006000D0F600000000D0000

00000000C0E00000B20CDF85500B02020000308B0F0B00220600

00000400E0A0084C9000B000000B20200000060006840D000000

00000000800900D000430000000E9E00007000408F000020F000

0000007C5070D0700030A00A000000000000000890540000C000

0000000005040000A0FA04F00000500000005D60DB0900F0AC40

00000D090000008E0600A0000008000B7700200000000C040000

00003000AF0000002070E0C00000F20000BB0000900072002A00

0000F0000A0006C0C60400D0200070300B2000000000B00E0000

0000020F0000A00C8000000000070000E70C8006000080F09000

00000006A000E90000000A0090000D0047DB00002E8000000FD0

0000200800000BA6004300000006000007000B00BC0700800000

000669C000002009300F00044000902080C0F05F0000E30604D0

00008004C060006BE00E5B0E000D6280CA0F000088E9BB00E000

00004000900008B7BA800A000000000470304000000000400000

0000005020B30400090604000000030D029705902E0707000000

0000000050D6000000C050F0200A04CCD0F05800B00D0B020200

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

00004C43758C6399846F0996A00F29DE953DA623EE4285068300

000C0B238303B46D4DD7F2DEF0092C7E255902A30E85D9246090

000220647F2B54DDED776B4E200B3E427C40472250BCDDEE0630

00059605493E4E0833D766B600099DA493335249E0C47ED05A70

0003C6F04E5C9FF34F0953F03002DA74F52B7EBA0723B40DF920

000A06960C2FBEEFD58DE4AAE00D75F9653907A7E22C800F32B0

000F875F8058834E5AC3B882F00B55333C2C6D78EF2B0A359650

000B5368030A277EEB40E76940047F32EFAA077C82E05ED8F380

000A84BA7AC06222BE5344CAE0039344292B078A0809694B20F0

0003549A680D03A5CA6A2320700EDCEE0F0D305D305363500200

000624E8D93DF033AB284797900DFE77330A748D0C905098DD70

0000CA3DC4F4030F40240E88C007FD864DE25AA0F5EF4FE3A650

0000DC5D864C2640964C27F7000CCA0C5568CE0A50CDA6A540B0

000487AB6868ED3403506CD37008B3DA8264C037EDE84A605700

000E52892E326BF620BDF26AC00FB09B77FF066D4C7CB7522230

0008FF0C607FE5CC4E026CB46000B5CDF64047BCF0C6606352C0

000B69862BBF6F29F8E07AB9700B0CA38A08D8A5CCBE5C4A8950

0003655EA30F84D49ED50D6AE008C5D2E084B323D39DCF2652A0

000974CB4588F50264A590E6600CCE3408205C0E580E7534A990

00087BF7868CAABF8B6D7307600990A0D258DB45F368C424CDB0

00089454DEE940433638852080009500C7FFFE72DF7027B96020

000D36C6F8BA4AB29AD2D3C200020024FCFB5D940C256CB7CFE0

000FA0A8BD6AA4786565224A500005AFE906B4BD42D8C5BDF2D0

000577090B5C2AA9F07FBE420000032FC3B95D3CF8507230E9B0

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000540829D655ED6F8BA869500084094372B7A9C9C488B5E5930

000A4F26D2FE7674E967E64020084F0B55AF0B0B437AF3E0D3C0

0008B40E2E5B9A2084C7680BF0050B20DA620D89FC94F83ABBE0

0007849736A2EDA5A45600CE600D9FB609B2A343AD5B84378AB0

0007D32F97A356AE7BC007E0F00FD3B8C0CA35AE0DBEA62AEB00

00020448822A5A24AB8042BF7002CBDE5D029436EF09D2328E40

00044CDF88A323E2FF06427BC00C47BF32B0B7FB3434A07442B0

000AA4097A5FD08D40BCA7B4E0070E5624A9020A7A30F04AB8E0

000D389BB56A3F6F05DBA22AE006899E29DEA0FED4808EC20FB0

000EED038E88C860FD7585CFA006B823DA97570942547E62D220

000468B3B930DA0730B07DBD200B0BB6CA8D8660830F08B2EA70

000D54B2B6C9B0F6F69BE2325005723B47F67F3C00B9BB6E26B0

0008D5967F7B068C85FDD0035008A3B8778EAECA60A2206866F0

00023FF9CA708FAEDAD5B779200E9EED809E43FC0F0F675DFFE0

00062777D90095E343AFF905400D064A28F6BDCBD55009CE4640

00038A7570529525C44AEE4D000DCE052D66BBBECCDF07AA99A0

000AE65900F65C757608BB85600369DE64F82D9C0F7880C40B70

0003CA703CD88820ABC66584F0006A2982E784673D8C590E3850

000CC0054689582AF2465A88E00EE9CE35F7CA85DBFDDBD0B790

000CE09FD7FC36E058D5706C40088AD82E7E22345D0B9F5C09A0

000F0654FDC5F24B3F90E92F800F7ACEE3A54740FF90FC5A0080

00008DF0B25089ABAEC935A7800024650F2003B88D5FD6E0F400

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

Random weight graph 3 example                                               Random weight graph 4 example

0000000000000000000000000000000000000000000000000000

0000055080800000000000D000009400000B30000D0000000000

000A05E8E000000000000000000000000000060C090000007000

0000000F900000D0000440300000090000A0004000080E0000B0

000000000D000000F0000000000E000A00000000000F02000000

0000D050000000000000000D000000000200000000E000090000

00000000005000073C0000F00005050200007000B000000F00D0

00000B000040000E000F00000000000006800000040600000050

000006000000000000000000000000009900000F005000000000

0000007402F0000003000000C000F00EC9000F000C0000097000

00005090000000000000000600000006000000000000000D0050

0000000000050000000F0E0F00000400002A0000080000000000

000000000000B0000000E0F30000009008C00000000000F08000

000000000000000000A003000000C00000E00000000000000000

000E00002000000000DB00020000000000507000027000305000

000D000000000000004C00000000000000000000000000000070

00000E00000DE000000060A60000000000000A30000000000000

00000B000000000000000F000000000000000000000E08000000

000004000C000000000000000000020500000000E00000000F00

0000B0000500000000500000000070000E600000000000000400

000000000000000020000000000700003020000000D00A07B000

0000000000000030F000000000004000000008D00B0000000000

0000080000000000000000000000000000000E005000000D7B00

00000000504000000000000040000002000000000F40000A5000

000000090000900000000600000000A070000000000006007400

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0003000000004A00060A000D0000000000200A050F00A0000000

0002600000000000F000000000000F00C0000003000000000000

00000000040F00000004000F00003000E000000000000000E000

0000000069006080000000004000000008000000000300007800

00000D000070080F00000E900000700000500000000040600000

000000A80000D000E0000300000000600000B0E0000000CB0000

00030000C00060000000000C00060040800080000A0004800000

0000000000B00007000000008000F00A0B80000F0060B0002000

00000FB0000000000000D0D060000000800000800B00700000D0

00080000D00000000000E000000DC0F000070000000002009000

0000A055000000000200707000090008000400000000005C0080

000000000003000000B0080000000000000B0003000000006000

0000000000000000000005900000000070009000900000003000

00000000000000000900000000000000830000070A0000000000

00002060090000000C0009F00000000000090D0000000000A200

000D0000A000000A0000C0200000000000000700070000000000

000090000040000C0009000000000000000A00000000000005E0

000000B00400000000000720700E00008B0000000E700C000000

000BF0000E0C0040B0000035000008000000660000AB00005000

000000000000000000003900000B000050760000000065000000

000000000064000084002D000000E00800060000A05000800000

0000000000900300400AE025000000000B0000D0002000900500

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000900000F090000000D00300000008BC0000000000000000000

0000000B4040000000007030000090000A0E0000000000000000

00000000C0000070000000000000000200000000000000000000

000000000B000000000000000000000000020D00000000000000

00007000E00000C0000800000000630000C00000000000000000

00000000000800000000000C0000000C00000E30000000000000

00040008F0800000050000F00000000000000000000000000000

000000000F0000A00D0500C00000F00400000000000000000000

0000040000000000E0000000000B0000D0000000000000000000

0000F000004000000E00050000000B0300404000000000000000

000000000000C0000080009900000050008090B0000000000000

0002000000600B00000000B60000000080000000000000000000

00000000000002D04000000450000000000000E0000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000870000000000300000000000A00A30CEF000000000000000

00000B0000000020000600C00000800E000BE700000000000000

00000000007000D0600000060000000009030000000000000000

00000000000A00000000000000000000000A0000000000000000

00000000000F060008000FE20000F0A090000000000000000000

000300000C0000000000000070000050E0000400000000000000

000000000A000000000000000000000000000000000000000000

0000006420000400000000000000000000000003000000000000

000000000E08000000000006B004000B000000E3000000000000

0000D90E00008000005000F00000003006007006000000000000

000060000003000000000000000B000000000060000000000000

000830000000300000000000000000000F000900000000000000

0000000000000500000020000000090000600000000000000000

00000000000000000302000B00000000707A0200000000000000

0000000D0000000000000005000000040A000000000000000000

000004000000090040400000000000000A000000000000000000

0000F06400E000000D00C000000F0000A0000600000000000000

000070000000007A00300000000000CA0B500700000000000000

0000000000000700B00020F0000000A800000000000000000000

00006000C000300000000000000030D00F20A300000000000000

00000040770B0000003000090000F00047009000000000000000

00000B000000000000000B5D0000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

Random weight graph 5 example                                               Random weight graph 6 example

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000E00209008000F000000000000000000000000000000000000

000000E009000600000000000000000000000000000000000000

0000602700940000000000000000000000000000000000000000

000000000000EE00000000000000000000000000000000000000

000000A0F020000A000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000A000000000000020000000000000000000000000000000000

00000000000000E0D00000000000000000000000000000000000

0000000300000000000000000000000000000000000000000000

000000000D0000F0000000000000000000000000000000000000

00000090A00004E0030000000000000000000000000000000000

0000C90A00000200000000000000000000000000000000000000

0006000000000000000000000000000000000000000000000000

0000002006070000000000000000000000000000000000000000

0000000023000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00000000006F60000000000000000000F0000000000000000000

000C000000000E003C00000CE000040340000000000000000000

000000000D0600000000B0005007000000000000000000000000

0000000D0000840500800900000D00A000C00000000000000000

000000000D000000000000000008F00000000000000000000000

0000000008002000000400000000A00000000000000000000000

0002C00700D6000080000D00C00000A000000000000000000000

000F00000C000000070000070000000000C00000000000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00000F0300050E000E000F000000000000000000000000000000

000000000000000E05004000000000000F000000000000000000

0000500000D00000000E0002000BE00000000000000000000000

000D50000000000000000000000000260A400000000000000000

0007000500400000000E07000000400000000000000000000000

0000000000600ED00BD000500000000C0B000000000000000000

000A0C0000000700000000000000000000000000000000000000

000000000005040090080000000E000004E00000000000000000

000000000000000000000006000000E004000000000000000000

0000000000000000085206063000800000000000000000000000

0000000000000504005F0000A00B006090C00000000000000000

0000000000000000000007000008000000000000000000000000

00000000000C0000000000000000000000000000000000000000

0000000000009F000D0006040000800D00800000000000000000

000040000D0AE00E0900000000000F0000000000000000000000

0000000000800000000000000000000000000000000000000000

00004000000000000040000B9000B0003B500000000000000000

0000070000600C0000004000000B000000000000000000000000

0000E000A000EB00000000F00000040000C00000000000000000

000000002000000700000DB00000000000000000000000000000

00000F0C000000BD00000000000D005000000000000000000000

0000C0000500060B000000000000000C00000000000000000000

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

Random weight graph 7 example                                               Random weight graph 8 example

0000000000000000000000000000000000000000000000000000

0000070B00BA8000A504E2090000060C000BC60003000C300700

00000CE06040B60004300006000B000B00800280F00000000000

000000F00006F004C0000000800B0000030200000A0940000DF0

00005C000009A0060DCE0002600002E03D0000A0080302600000

0000000000A00000000C000000000E5D00040089007070000420

000003000D0000DE0F00E00020000E0B050000000E0000000B00

00000300A0E9F02A529300F0000BB4E0003C0B0000AA00057620

0007005E200C03700000D00E400E530000002069B0C0F0000080

00009CFF00007DE00D0B07C2000A000A40B00000050000000020

000A000090000C00000002B05000000B02000075000007000000

00004000000000000600000B00000D00400202A0040040000720

000F705008D0000070E0300F00060000000009000B00F020A020

00000BF002000770000000F00000F06008800000400800009000

000B0D0704900AB000040B000000000C005A5000900D000B0000

000B0000785040000005000040030D00BAF506000000000000F0

0000A08C99000004C20400C0000F00000000003A0F0B03B00800

00080D00000F0B00000070E000000300000024B00E0D0A000000

0000700009E6F000B0050060F000660000000009005000000340

00080000006B3E00400090200000003008C00003009C94B00400

000929F807640F003000F0000000003000000000D0C0200070D0

00000050E60A6FD7CF028040000450000A0D090000004E0E0000

0000C000B00000000080000D0000000800E00300000000E00000

000D000066A603A006500000000000B0B2004000050B000D3000

000000000CB400D00600000D0000000BE6B8B050000E00000BD0

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

000B0008F0AF0D60008300000007000900000A4089F09400B0D0

000B000009007455B0000E00E0009000EF805BD05E03005BE070

00000F0E400000800B40C800200E0080000D00080000A0A08000

000066DC0068C00003C20000000047000D0000BF088B90000040

0000009000C00700A00000000000407B00200500000060000900

00000009720004006020000E000E0006000D0F600000000D0000

00000000C0E00000B20CDF85500B02020000308B0F0B00220600

00000400E0A0084C9000B000000B20200000060006840D000000

00000000800900D000430000000E9E00007000408F000020F000

0000007C5070D0700030A00A000000000000000890540000C000

0000000005040000A0FA04F00000500000005D60DB0900F0AC40

00000D090000008E0600A0000008000B7700200000000C040000

00003000AF0000002070E0C00000F20000BB0000900072002A00

0000F0000A0006C0C60400D0200070300B2000000000B00E0000

0000020F0000A00C8000000000070000E70C8006000080F09000

00000006A000E90000000A0090000D0047DB00002E8000000FD0

0000200800000BA6004300000006000007000B00BC0700800000

000669C000002009300F00044000902080C0F05F0000E30604D0

00008004C060006BE00E5B0E000D6280CA0F000088E9BB00E000

00004000900008B7BA800A000000000470304000000000400000

0000005020B30400090604000000030D029705902E0707000000

0000000050D6000000C050F0200A04CCD0F05800B00D0B020200

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD

0000000000000000000000000000000000000000000000000000

000000000980000090070000000B000000000209003070002000

00000000E060D07022800000000607080700000CB00000635000

00003000500C004070CE0000000B2D005020002A000000000000

0000000040E020C02E000006C0000C0080000000E00750A00CA0

000200C0000000060AA500300000D0D0000007B0000070000800

00000000000000A00030070A0009B60000B0030D000000000000

0000000000000000000000050000000000000007003000009000

00000D00000004700A007000000000088006002D0200B5800000

0000000000D07BC0000E0B0000000000000F0070D000000A0000

0000F080000909D60042700B00000407C0000C0000000000B020

000900002000000D5C3B0200800200600B00000000A0000F0400

00000000CE0902000000000040000C0060009E3000000E460090

000903000600C0900074007000000DD000000E00000000C86000

000000000D0B00570A0424F000000040A7003000000000000400

000CA00000D00000000FF00B000000F0000000000B0000000500

0000C000000300E0C00E092000000E0000700000020680000000

0000282D0000C04000000000000C000A0000000B0000E0E0B000

0000B00009B0F00D00000F00000000800005420020D008000000

000B00D006000400004040000000000000C0500700008500C000

000005D070030000000073050000008000400000000000000000

000200009060000A0000000000030000066000C0C0B20F000000

000000C200E0900000060500000A00700F80E0E06E0000000000

000000000F000C0000500000000006B00000B060000000900000

00000000090039080000D00A000000E002000000E00000000000

0000000000000000000000000000000000000000000000000000

0000000000000000000000000000000000000000000000000000

00000502040000B0000004CB00000000F006EA00000700000A00

000B000080050000000000A00000F000000000B0000809000000

0000E000BF000E0E000300000000E0D0E9C000C000000DD00000

000000082002FA0070000000000000D0040C60AE00950008AC00

00000056080A0090C00000623000030500908000030090D00000

00060EE00000007000000000000500002000000B008D00000700

0000000000002B00000004000000000B0E0000D4090000000400

000000E000000A008000000000050EC09400000040E0000D3550

00003E04000000000D090CB00000009C05000000D00900408000

000C000000000000E000000A000000000C00000B0390F0600500

000000000008000000F70000D0000E0000200B00050003000000

00005B00000200000000900000000000000A0000000000069500

00000A0000B00005D00B00400000030390F0009D000060000000

000080000300000A00099000000040000A090000000600000000

0009C4006000D000000800020000900200F62000220000B03070

00050000D00000900903900F200000007C000040000C0E000EB0

00000004000E0609000B000FC00E0700000000800A09000000A0

000000000400700070005000500A6D0095000000000A070C0000

000D0B0000300006060040D7000877F090000000000000000800

000000000B00000B0002000000000030050D0000000000300000

0006000000F00C000C0E0400000073005AE90300000000700000

0000300500000B80004000080000C0F00B02B0000000060D0E00

0000000000000000000000000000000000000000000000000000

00AAAAAAAAAAAAAAAAAA0111000000FF001F0003000F000300FD